METU – EEE

Middle East Technical University – Electrical Electronics Engineering Department

PROJECT REPORT

by Serhat ÖZKÜÇÜK

within the scope of the course

EE568

SELECTED TOPICS ON ELECTRICAL MACHINES

by Dr. Ozan KEYSAN

2019 – 2020 Spring Semester

PROJECT REPORT NO	: 02

PROJECT NAME		: Motor Winding Design & Analysis

ASSIGN / DUE DATE		: 18.03.2020 / 31.03.2020 , 23:59

Introduction

In this report, we assume have a 20-pole, 120 slot, 3-phase machine for integral slot winding and for 20 or 22 pole number, we choose a slot number between 20 and 30 for fractional slot winding. Integral and fractional slot windings are investigated and designed. Also 2D FEA model of the fractional winding is simulated and analyzed. Winding diagrams are drawn. Distribution factor and pitch factor of the each winding are calculated. Also, 3rd and 5th harmonics winding factors are calculated.

1. Integral Slot Winding Design

We assume we have a 20-pole, 120 slot, 3-phase machine.

q is the slot number per pole per phase:

q=12020×3=2 slot/pole.phase

α is the electrical angle between two consecutive slot:

α=360o120×202=30oelectrical

Assume each coil has 1 turns and full pitched. So, winding will be single layer.

| slot | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| winding | A | A | -C | -C | B | B | -A | -A | C | C | -B | -B |

Fig. 1: Winding diagram of the given integral slot machine for one pole pair (12 slot)

Winding Factor

2. FEA Modelling (2d Linear Materials)

For the given geometry, the FEA model is constructed in Ansys/Maxwell (see fig.9). Stator and rotor materials are selected as steel_1008 in Ansys/Maxwell library. The material property is set as constant permeability as µ=902.6.

Fig. 9: FEA model of the given geometry in Ansys/Maxwell

System analysis is done for three positions as rotor angles are 0o, 45o and 90o. For these three position, the flux vectors, magnetic loading, inductances and energies are calculated.

2a) Flux Density Vectors for Linear Materials (0, 45, 90 degree rotor)

(a) 0o position(b) 45o position(c) 90o position

Fig. 10: Flux lines and vectors for linear (µ=902.6) stator and rotor material

Fig. 11: Magnetic loading (B) distribution of the system with linear (µ=902.6) stator and rotor material

2b) Inductances and Stored Energy in the System

Inductances in the system is obtained as shown below. These values are too low with respect to analytical calculations maybe because of material depth is assumed as too low in 2D analysis.

CoEnergies of the system for 0, 45 and 90 degree positions,

Wcoenergy=12Li2

0o position    1219.10×10-6×32=8.595×10-5 J

45o position    1212.67×10-6×32=5.701×10-5 J

90o position    126.43×10-6×32=2.893×10-5 J

These values are meaningless for to compare with analytical solution but we can compare the linear and non-linear material difference. We expect that for linear material coenergy and energy of the system are equal each other but for non-linear material (non linear BH curve) coenergy is slightly smaller than energy.

2c) Torque Generation in the System

Torque data of the system is recorded for three specific angle (0, 45 and 90 degree). So the torque graph is looks like triangular but actual form must be sinussoidal. Max torque is generated at 45o angle position.

Fig. 12: Torque – position graph and actual values for linear material

3. FEA Modelling (2d Nonlinear Materials)

In this section, stator and rotor materials are selected as again steel_1008 in Ansys/Maxwell library. But, the material property is set as nonlinear characteristic as nonlinear B-H curve that is given in Ansys/Maxwell library.

Fig. 13: Non-linear B-H curve of steel_1008 material in Maxwell

3a) Flux Density Vectors for NonLinear Materials (0, 45, 90 degree rotor)

(a) 0o position(b) 45o position (c) 90o position

Fig. 14: Flux lines and vectors for no- linear (steel_1008 nonlinear BH curve) stator and rotor material

Fig. 15: Magnetic loading (B) distribution of the system with non-linear (steel_1008 nonlinear BH curve) stator and rotor material

3b) Inductances and Stored Energy in the Nonlinear System

Inductances in the nonlinear material used system is obtained as shown below. These values are too low with respect to analytical calculations maybe because of material depth is assumed as too low in 2D analysis.

CoEnergies of the system for 0, 45 and 90 degree positions,

Wcoenergy=12Li2

0o position    1219.01×10-6×32=8.554×10-5 J

45o position    1212.67×10-6×32=5.701×10-5 J

90o position    126.39×10-6×32=2.875×10-5 J

When we compare these result with respect to linear material used result, we can see the coenergy of the system decreases slightly because of the nonlinear characteristic of BH curve. For energy comparison, the situation is vice versa.

3c) Torque Generation in the Nonlinear Material System

As expected, maximum torque value at 45o position is slightly smaller than linear material based system because of the inductances. Inductances of the nonlinear based system are obtained slightly smaller than linear based system. Torque expression contains Ld-Lq statement. So, for nonlinear based system, this difference is slightly bigger than linear system.

Fig. 16: Torque – position graph and actual values for non-linear material

3d) The effects of fringing and saturating effects with the linear and non-linear materials

The difference between linear and nonlinear materials based system can be seen from given figure below that shows the magnetic loading (B) distributions in same scale for 0o position.

(a) Linear stator and rotor material                      (b) Non-linear stator and rotor material

Fig. 17: Magnetic loading distributions at 0o position for linear and non-linear BH curve materials

For linear material, the magnetic permeability of the material is constant all location on the geometry. So, magnetic loading increases at the inner corners of the stator because of the small length of flux paths. But, the material tries to distribute this loading to homogeneous. In non-linear material, magnetic permeability is constant up to 1.4T, after that value nonlinear region starts and permeability decreases. So, inner corners of the stator expose this non-linear region and permeability is low in these areas and magnetic loading saturates.

4. Control Method

Beacuse of the saliency of the rotor, rotor tries to come to position 0o (180o) position (figxx-b3) for minimizing the reluctance. So, in that position there will not any excitation (avoiding the locking rotor). In this manner, we have to excite the system with maximum energy at near of 90o (270o) position of the rotor. After this excitation rotor gains speed and tries to keep it with its inertia up to next maximum excitation at 270o (90o) position. The excitation wave form with respect to rotor position angle θ is given in figure below.

(a)

(b)

Fig. 18: (a) Stator excitation signal for control the rotor (b) Rotation motions with flux lines of the control method

With this logic, we can design a basic speed controller for this machine with PID controller as shown below. There is no ready to use motor model exactly same as our model, So, I used the this general model with torque input. Driver function controls the two seperated switch that can also connect the DC source for creating single phase signal as we designed. So the stator excited when the rotor at position every 90o and 270o position.

Fig. 19: Designed control system in MATLAB/Simulink

The generated torque is shown in figure below that obtained by Maxwell with designed excitation. As expected, maximum torques are obtained at 135o (also continues periodically at 45o 135o 225o 315o).

Fig. 20: Generated torque from FEA analysis when the excitation is derived from designed control system

5. Motion Animation

Animation videos are added on github.

https://github.com/SerhatOzkucuk/EE568-Selected-Topics-on-Electrical-Machines/blob/master/ee568_ozkucuk_control_method_animated.avi

https://github.com/SerhatOzkucuk/EE568-Selected-Topics-on-Electrical-Machines/blob/master/ee568_ozkucuk_dcexcitation_animated.avi

# REFERENCES

1. https://github.com/odtu/ee568

2. http://keysan.me/ee568/