METU – EEE

Middle East Technical University – Electrical Electronics Engineering Department

PROJECT REPORT

by Serhat ÖZKÜÇÜK

within the scope of the course

EE568

SELECTED TOPICS ON ELECTRICAL MACHINES

by Dr. Ozan KEYSAN

2019 – 2020 Spring Semester

PROJECT REPORT NO	: 04

PROJECT NAME		: PM Motor Comparison Analysis

ASSIGN / DUE DATE		: 03.06.2020 / 28.06.2020 , 23:59

Literature Review

In this study, 1kW permanent magnet synchronous machine (PMSM) type servo motor design aspects are discussed. In this concept, several comparative designs are modelled with FEA performance considerations. Servo motors are essential machines in industrial applications and have a basic specific property as high torque with low inertia (sausage type machine) for acceleration, speed, position accuracy and robust performance. Providing the basic properties depends on machine parameters such as diameter of the rotor-stator, pole-slot number, slot dimension, winding types, permanent magnet type etc. Design of these parameters effect the magnetic and electric load distribution, air gap flux density distribution (harmonic contents), flux leakages, cogging torque, end winding losses, thermal performance, machine dimension and cost [flieh].

The first critical consideration for designing AC PMSM servo is magnet type, location and shape in the rotor structure. There are two types PMSM servo known as internal permanent magnet synchronous machine (IPMSM), and surface mount permanent magnet synchronous machine (SMPMSM). The main difference between them the IPMSM can produce extra reluctance torque with hard manufacturing process. The detailed comparative performance analysis of the two type machines under identical conditions in terms of air gap flux density distribution, back EMF and current waveforms is critical for design process. IPMSM has more harmonics in its air gap flux density waveform than SMPMSM, which is caused by the rotor openings and the smaller air gap. Back EMF wave forms of the two type machines at the rated speed have nearly same amplitude. However, there are more harmonics components in the IPMSM, which are caused by the more harmonic air gap flux density components. The harmonic components of armature currents are slightly higher for the SPMSM with respect to IPMSM, which is due to the lower inductances. IPMSMs are superior in costs while SPMSMs are more robust, with higher overall efficiency and cooling performance. [dong].

Another important design issue about permanent magnets is shaping.

1. Analytical Calculations & Sizing

In this project, we design a AC PMSM type servo with respect to following specifications:

The application: Industrial servo systems

The type of the machine: Permanent magnet synchronous machines (PMSM), surface mount type.

Power, voltage and current ratings:  1kW, 220V - 4A (nominal for 1 phase), (3 phase inverter ratings: DC level of bulk is 310V (from 220V AC rms), 190Vph-ph, 3.1Aph, Y connected)

Operating conditions: 3000 rpm rated speed, 4Nm rated torque (12Nm max. torque), Duty type:S1 (Continuously), Natural cooling, IP 54 Enclosure (industrial applications), 0-55’C ambient temperature.

Limitations  (if there is any) such as mass, diameters, cost, efficiency: max. mass 6kg, low inertia for obtaining dynamic response so the dimensions will not exceed the motor length L_max=250mm, outer diameter Do_max=125mm. (The smaller it can be designed, the better.)

Design Procedure (Analytically)

Machine constant: 	Pmech out=CmechD2Lfsyn

Where, Pmech out is output power,  D2L is represents the rotor volume, fsyn is synchronous frequency and Cmech machine constant is :

Cmech=π22kwAB

Where, kw is winding factor, A is electric loading and B is magnetic loading.

AAm=Nturn per slotIQπDi     ,     B(T)=pφpoleπDiL

Where, Nturn per slot is turn number per slot, I is current flow through wire, Q is slot number, Di is inner diameter, L is length of bore,  p is pole number, φpole is flux per pole.

If we assume the iron core material for stator and rotor we can select the max magnetic loading as 1.8T (1.2T for average) which is saturation limit for selected material. Selection of electric loading depends on the required torque level.

Torque=π×Di×L×σFtangentialaverage x×rrotor=4Nm

Where, σFtangentialaverage (x) is tangential stress at x distance (will be rotor edge),  rrotor is radius of the rotor.

σFtangentialx=BsinxAsin(x)

σFtangentialaverage x=0.5×AB

According to specification of our motor the rated torque is 4Nm (12Nm max.) at rated output power 1kW. Also, max outer diameter is 125mm, so rough calculation (using Do/Di table with respect to the worst case pole number 2) we can select the rotor diameter as nearly half of outer diameter 60mm (considering air gap, outer cover etc.). Determination of the axial length L of the machine can be estimated by using again rough definition of aspect ratio for servo motors as LD≫1(Torque-Aspect ratio graph from METU EE568 courses – Machine Design Basics, page 35), we can select the LD=2. At this time L will be 250mm. The air gap can be selected as 1mm for providing mechanical reliability.

If we combine the known parameters up to now:

Torque=π×0.06×0.25×0.5×A×1.2×0.03=4Nm

The average electrical loading will be 4715 A/m, (4.715 kA/m).

Pmech out=π22kwABD2Lfsyn=1kW,   fsyn=50Hz , kw=0.79

The air gap can be selected as 1mm for providing mechanical reliability. The slot and conductor number can be determined as (nominal current from spec is 4A):

AAm=Nturn per slotIQπDi=4715=Nturnperslot×4Qπ×0.06→ Q×Nturnperslot=222.18 total cond.

According to these calculations we know that the total conductor number in the stator. So we can determine possible pole-slot combinations with respect to our winding factor result using the table 1.

Table 1: Winding factors for different combinations of pole and slot numbers and double layer winding (METU EE568 courses – Airgap & Mechanical constraints, page 42)

For determining the slot number Q, we can use the formula as follows and determine the q value which is slot per pole per phase. Where the  m is phase number, p is pole number. For improving the smooth torque performance we can select the q as fractional.

Q=m×p×q

If we select a suitable conductor which can carry 4A current from AWG table, the #20 conductor (area: 0.518mm2) is suitable for our design. But, the machine max torque will be 12Nm (3xNominal torque) so, we must select the conductor which is capable of carry 12A max. current. This is the #14 conductor for safe design (area: 2.08mm2). We can assume the slot fill factor 60%.

Fig. 1 :AWG table and selected conductor.

If we create a table which is shown the possibility of the design and mechanics with respect to fig. 3, we can obtain table 2.

Table 2: Design mechanics control table for different pole slot selection.

| Pole number | Slot Number | Winding factor | N turn per slot | Slot area* mm2 | Possibility |
| --- | --- | --- | --- | --- | --- |
| 4 | 6 | 0.866 | 37 | 470 | OK – 135 turn/slot |
| 10 | 12 | 0.933 | 18.5 | 236 | OK-68 turn/slot |
| 16 | 18 | 0.945 | 12.33 | 157 | OK-45 turn/slot |
| 20 | 21 | 0.953 | 10.57 | 134 | OK-38 turn/slot |

*slot area is calculated roughly with respect to rectangular parallel teeth and slots. Slot height is assumed 20mm (back core 10mm). Slot number calculation is done with taking the mid-point of slots. Slot fill factor 60%. Single conductor area is 2.08mm2.

From the table 2, all possibilities are reliable when back core is 10mm. So according to flux density distribution and saturation we can increase the back core or if back core flux density is under the saturation level, we can increase the rotor diameter (preventing leakage) or decrease the machine dimension (decreasing slot height). The rotor diameter is 60mm (circumference:18.84cm). If we placed 10 magnet on the rotor, we have 1.8cm place. It is the most suitable design because magnet size is bigger than 1cm (it is possible to find from the market). So we can select the 10 pole 12 slot machine for implementation. 4 pole 6 slot machine has low winding factor so there will be much more harmonics. 16 pole or 20 pole machine implementation can be hard because magnet size will be lower than 1cm (brittle magnets).

If the assume slots gets wider with diameter slots (rectangular teeth), the d ratio can be assume as 0.7. This means that the slot height is:

d=0.7=60mm85.7mm→slot height=12.85mm

Fig. 2 : d ratio on the machine and Do/Di design guide

According to design table of T.Miller (fig. 2), the outer diameter Do is:

for 10 pole machine→DoDi=1.54,  Di=60mm,  which yields

Do=92.4mm,  Back core=3.7mm

(a) 4 pole 6 slot, 0.866 winding factor                      (b) 10 pole 12 slot, 0.933 winding factor

(c) 16 pole 18 slot, 0.945 winding factor                  (d) 20 pole 21 slot, 0.953 winding factor

Fig. 3 : Possible slot dimensions, back core thickness, stator slot design mechanics.

Material selection is another issue for design. If we calculate the flux density from the selected magnetic loading data as follows: (Assuming 10 pole machine)

B(T)=pφpoleπDiL=1.2=10φpoleπ×0.06×0.25  ,  φpole=0.00565 Wb/m2

We can determine the magnet flux density Bm as follows:

φtotal=pφpole=Bm×Am

Where Am=2π(rrotor+magnet thickness)×L is area of the magnets which is assumed the magnets are separated the rotor surface (magnet thickness is assumed as 5mm).

Bm will be 1.02T. According to this value we can select the magnet as N42 neodymium, under 40oC operating temperature with permeance coefficient 4, as shown in fig. 4.

Fig. 4 : Load line on the B-H characteristic of the given Neodymium magnet, Permeance Coefficient (Pc): 4, B: 10.08 kilogauss, H: -2.52 kilooersted, |BH|: 25.39 mega-gauss-oersted

The electrical parameters of the design can be selected as 190Vph-ph, 3.1Aph for Y connected machine. Total conductor number was 222.18 that is calculated previously so total length of the conductor will be 222×L×2=111m. The selected AWG14 conductor has 8.286mΩ/m resistance. So the phase resistance will be 8.286×1113=306mΩ. Rated speed of the machine will be nratedrpm=120fp=600rpm  at 50Hz with pole number 10.

Analytically designed machine is given in table 3.

Table 3: Analytically designed machine parameters.

| Pole number p | 10 | Slot height | 12.85mm |
| --- | --- | --- | --- |
| Slot number Q | 12 | tooth width | 9.1mm |
| Winding factor kw | 0.933 | Back core | 3.7mm |
| Slot /phase/pole q | 2/5 | Magnet | N42@40C, 1.008T |
| Outer diameter Do | 92.4mm | Flux per pole φpole | 5.65x10-3 Wb/m2 |
| Inner diameter Di | 60mm | Phase Voltage,Current | 190V,3.1A, 600rpm |
| Air gap | 1mm | Phase resistance | 306mΩ,AWG14 |
| Axial length L | 250mm | Power, Torque | 1kW, 4Nm(12Nm max) |

2. FEA Modelling

Analytically designed machine in part 1, is simulated in this part using Motor Analysis Software (FEA), MotorXP-PM. The constructed geometry is given in fig. 5.

Fig. 5: FEA model of the designed machine in Motor Analysis Software MotorXP-PM

The FEA model is constructed with respect to calculations in part1 and obtained table 3. Machine slot configuration is selected as parallel tooth and layer orientation is selected as left/right to prevent the skin effect and flux saturation. Magnets are located on rotor surface (SMPMSM) because easy production and high performance (details are in literature review part) (fig. 6.).

Fig. 6: Stator and rotor geometry details.

Stator and rotor materials are selected as M-19 lamination steel and magnet type are N42 neodymium.  The conductors are copper. Total machine weight is 7.68kg with respect to designed geometry and material selection as shown in fig. 7.

Fig. 7: Selected materials for stator, rotor, magnets with suitable stacking factors and weights of the materials and machine.

The machine slot per phase per pole was 2/5 fractional slot. So the winding configuration of the machine is given in fig. 8. AWG#14 conductor is selected for this design and slot fill factor is assumed 60%. Totla conductor number was 222 and 18.5~19 conductor is located per slot. Winding factor was calculated as 0.933. According to our calculation phase resistance without considering end windings was 306mΩ. If we consider the end windings the phase resistance will be 341mΩ as shown in fig. 8.  Meshing the machine is given in fig. 9 for FEA analysis.

Fig. 8: Winding configuration of designed machine

Fig. 9: Meshing the design for FEA result

The drive parameters of the design are given in fig. 10. Space vector PWM drive method is selected with 10kHz switching and IGBT switches (IRG4PC50FD_IGBT). The DC bus voltage is set as 310V (rectified 220AC).

Fig. 10: Driver settings of the machine

FEA Analysis Results

Design is driven under rated parameters (rated torque 4Nm, rated speed 600rpm). The electrical and magnetic results are given in fig. 11 - fig. xx.

Fig. X: Magnetic flux density distribution

Fig. X: Simulation results, electrical, machine constants, flux density levels

Fig. 11: Phase A current, Voltage and Back EMF

Fig. 12: Cogging torque and Electromechanical torque

Fig. X: Air gap magnetic flux distribution (Wb), MMF waveform and radial force distribution (N)

Design Improvements

As shown from the flux distribution, the back core thickness is not sufficient for saturation. Also slot openings corners have near of the saturation level flux density. So, the machine will be improved by mean geometrically. The electrical results and performance is acceptable for required specification data. Revised geometry is in fig.xx.

Rotor dimension is suitable. So we must change slot height and tooth width. The revised d ratio can be assumed as 0.75. This means that the slot height is:

d=0.75=60mm80mm→slot height=10mm

According to design table of T.Miller (fig. 2), the outer diameter Do is:

for 10 pole machine→DoDi=1.54,  Di=60mm,  which yields

Do=92.4mm,  Back core=6.2mm

The revised design:

| Pole number p | 10 | Slot height | 10mm |
| --- | --- | --- | --- |
| Slot number Q | 12 | Slot width | 8mm (fill f.~ 0.7) |
| Winding factor kw | 0.933 | Back core | 6.2mm |
| Slot /phase/pole q | 2/5 | Magnet | N42@40C, 1.008T |
| Outer diameter Do | 92.4mm | Flux per pole φpole | 5.65x10-3 Wb/m2 |
| Inner diameter Di | 60mm | Phase Voltage,Current | 190V,3.1A, 600rpm |
| Air gap | 1mm | Phase resistance | 306mΩ,AWG14 |
| Axial length L | 250mm | Power, Torque | 1kW, 4Nm(12Nm max) |

Max. Loading & Half Loading

The servo machine can handle a high torque (3xrated) in a short time for some applications. In the design specifications this max torque level is 12Nm at 1kW. In fig. X, the max loading simulation and in fig. X, half loading simulation results are given.

Inductances

Design Improvements

Torque

3. Comparison & Discussion

In this section,

3a) Flux Density Vectors for NonLinear Materials (0, 45, 90 degree rotor)

curve) stator and rotor material

3b) Inductances and Stored Energy in the Nonlinear System

3c) Torque Generation in the Nonlinear Material System

As expected,

3d) The effects of fringing and saturating effects with the linear and non-linear materials

The difference

4. Conclusion

Beacuse

# REFERENCES

APPPENDIX